# Antigravity IDE Handoff: ArduPilot-Based Operation Touchdown System

# Purpose

Build a competition-ready **ArduPilot-based UAV/UGV collaborative autonomy system** for the Raytheon AVC “Operation Touchdown” challenge using the exact hardware below. The system must support:

UAV autonomous launch from the UGV.

UAV scouting and finding the destination ArUco marker.

UAV sending destination information **directly to the UGV without a ground station**.

UGV driving to the destination.

UAV landing on the **moving** UGV.

Entire system arriving at the destination.

No GPS required for the UAV landing flow.

This prompt is intended for an AI coding assistant that will write the full implementation.

---

# Hardware already chosen

## UAV

Cube Orange+

Hexa X airframe

Jetson Nano companion computer

ZED X camera mounted downward

Garmin / PulsedLight LidarLite v3 downward

Holybro H-Flow optical flow sensor

ESP32-S radio module for direct UAV↔UGV link

## UGV

ESP32-S radio module for direct UAV↔UGV link

Existing UGV autopilot/control stack and drive computer are already being coded around MAVLink/DroneKit-style control

## Existing uploaded code to reuse/adapt

`mission1.py` — rough UAV mission script prototype

`v2v_bridge.py` — serial framing + ESP32 bridge protocol

`esp_uav.cpp` — ESP-NOW bridge firmware for UAV-side ESP32

`esp_ugv.cpp` — ESP-NOW bridge firmware for UGV-side ESP32

`ground_station.py` — rough UGV motion control prototype

Treat these files as **starting references**, not finished production code.

---

# Competition requirements extracted from the uploaded rules PDF

These are non-negotiable system constraints.

## Core scenario

The UAV must autonomously launch and scout the field containing **1 destination ArUco marker** and obstacles.

The UAV must **directly communicate** the destination to the UGV **without using a ground station**.

The UGV then travels to the destination while avoiding obstacles.

The UAV lands on the moving UGV and continues with it to the destination.

Both UAV and UGV must behave **fully autonomously**.

## Challenge timing and motion constraints

Minimum UAV altitude during challenge flight: **4 feet**.

Minimum UAV flight time: **5 seconds**.

For Challenge 2 and 3, the UAV must land on the moving UGV within **10 minutes from first UGV movement**.

After landing, the UAV must remain with the moving UGV for **10 seconds** without separating.

UGV minimum speed: **0.2 mph**.

## Destination and obstacle constraints

The destination is the space within a **5-foot radius** circle from the center of the designated ArUco marker.

Challenge 3 obstacles are real 3D objects the UGV cannot drive over.

Obstacles may include cardboard boxes, traffic cones, and buckets.

Minimum obstacle spacing: **5 feet**.

## Operational and management constraints

Teams must document a **requirements matrix** and **test documentation**.

Code and design should be easy to adjust for field conditions.

The rules explicitly warn teams to test ArUco detection distance, angle sensitivity, and lighting robustness.

---

# Design decision: use ArduPilot precision landing, not RC override flight logic

The uploaded `mission1.py` currently uses RC override / throttle pulses and a rough vision loop. Do **not** keep that as the main architecture.

The UAV stack must be rewritten around:

**ArduCopter on Cube Orange+**

**Precision Landing using MAVLink `LANDING_TARGET` messages from Jetson Nano**

**Optical flow + downward rangefinder for low-altitude GPS-denied stability**

**ArUco-based relative target estimation on the Jetson**

**Direct UAV↔UGV communication through the ESP32 bridge**

The UGV stack may continue to use a MAVLink/DroneKit-style control program if that is already your path, but the interface should be cleaned up and formalized.

---

# High-level architecture

## UAV responsibilities

Boot Jetson services.

Connect to Cube Orange+ over MAVLink.

Acquire camera frames from the downward-facing ZED X.

Detect the destination ArUco on the field.

Estimate the destination location relative to a chosen frame.

Send destination payload directly to the UGV over the ESP32 bridge.

Track the moving UGV landing marker.

Send `LANDING_TARGET` updates to ArduPilot at a high rate during the landing phase.

Command ArduPilot modes cleanly: arm, takeoff, navigate/search, land.

Log all detections, communications, and mission state transitions.

## UGV responsibilities

Receive destination payload directly from the UAV.

Acknowledge receipt to the UAV.

Drive toward the destination while avoiding obstacles.

Broadcast motion state, speed, and optional heading back to the UAV.

Expose a stable landing deck with a dedicated landing ArUco marker.

Continue moving after touchdown long enough to satisfy ride requirements.

## ESP32 bridge responsibilities

Provide direct, low-latency UAV↔UGV messaging.

Frame packets over serial using the existing `0xAA + type + len + payload + xor` protocol.

Forward telemetry, commands, and debug messages over ESP-NOW.

Support reliable heartbeats and sequence numbering.

---

# Physical marker strategy

Use **two different marker roles**.

## Marker A: destination marker on the field

This is the competition destination marker placed by judges.

The UAV must detect it during scouting.

The UAV sends the destination information to the UGV.

## Marker B: landing marker on top of the UGV

This is your own landing fiducial mounted flat on the UGV landing deck.

It should be large, high-contrast, and easy to detect while the UGV is moving.

The UAV uses Marker B for moving-platform precision landing.

Do not reuse the same mission logic for both markers. Use separate IDs and separate state-machine roles.

---

# Key implementation assumptions

These assumptions should be made explicit in code/config:

**ArUco dictionary**: use a configurable dictionary and marker IDs.

**Camera**: downward-facing ZED X, calibrated, with saved intrinsics/extrinsics.

**Coordinate frame**:

- Use vehicle body frame for `LANDING_TARGET` position fields.

- Prefer full `x, y, z` body-frame landing target when pose is available.

- Fall back to `angle_x`, `angle_y` only if needed.

**Altitude source**: LidarLite v3 is the primary low-altitude height-above-ground sensor.

**Flow source**: Holybro H-Flow provides optical flow; onboard lidar from a flow unit should not be the primary altitude sensor if a better lidar is available.

**Moving target support**: ArduPilot precision landing must be configured for a moving landing target.

**No ground station in mission logic**: direct UAV↔UGV messaging only for competition mission data flow.

**Safety**: emergency stop, comm-loss handling, and target-loss handling must be implemented.

---

# Required software deliverables

The AI should generate a full project tree like this.

operation_touchdown/

README.md

requirements.txt

pyproject.toml

config/

airframe.yaml

camera.yaml

markers.yaml

mission.yaml

mavlink.yaml

radio.yaml

ugv.yaml

docs/

requirements_matrix.md

test_plan.md

wiring.md

field_ops_checklist.md

common/

__init__.py

logging_utils.py

math_utils.py

frames.py

time_sync.py

health.py

radio/

__init__.py

v2v_protocol.py

v2v_bridge.py

message_types.py

heartbeat.py

uav/

__init__.py

main.py

state_machine.py

mission_manager.py

mavlink_client.py

ardupilot_control.py

precision_landing.py

takeoff.py

search_pattern.py

destination_detection.py

ugv_tracking.py

camera/

__init__.py

zedx_camera.py

calibration.py

vision/

__init__.py

aruco_detector.py

pose_estimator.py

landing_target_encoder.py

target_filter.py

sensors/

__init__.py

rangefinder_monitor.py

optical_flow_monitor.py

safety/

__init__.py

failsafes.py

watchdog.py

ugv/

__init__.py

main.py

state_machine.py

motion_controller.py

mavlink_ground_vehicle.py

destination_receiver.py

obstacle_avoidance.py

telemetry_sender.py

deck_status.py

esp32/

uav_bridge/

platformio.ini

src/main.cpp

ugv_bridge/

platformio.ini

src/main.cpp

scripts/

run_uav.sh

run_ugv.sh

calibrate_camera.py

print_marker_pack.py

test_radio_link.py

bench_test_landing_target.py

replay_log.py

tests/

test_protocol.py

test_aruco_pose.py

test_state_machine.py

test_landing_target_encoding.py

test_config_loading.py

---

# Mission state machine to implement

## UAV state machine

BOOT

-> WAIT_FOR_AUTOPILOT

-> WAIT_FOR_RADIO_LINK

-> PREFLIGHT_CHECKS

-> ARM

-> TAKEOFF

-> SEARCH_DESTINATION_MARKER

-> ESTIMATE_DESTINATION

-> SEND_DESTINATION_TO_UGV

-> WAIT_FOR_UGV_ACK

-> SEARCH_UGV_LANDING_MARKER

-> TRACK_MOVING_UGV

-> PRECISION_LANDING_APPROACH

-> TOUCHDOWN_CONFIRM

-> RIDE_ALONG_TIMER

-> MISSION_COMPLETE

FAILSAFE states:

COMM_LOSS

TARGET_LOSS

RANGEFINDER_BAD

CAMERA_BAD

EKF_BAD

ABORT_LAND

EMERGENCY_LAND

## UGV state machine

BOOT

-> WAIT_FOR_RADIO_LINK

-> PREFLIGHT_CHECKS

-> WAIT_FOR_DESTINATION

-> ACK_DESTINATION

-> START_MOTION

-> NAVIGATE_TO_DESTINATION

-> AVOID_OBSTACLE

-> EXPECT_UAV_TOUCHDOWN

-> TOUCHDOWN_CONFIRMED

-> CONTINUE_TO_DESTINATION

-> STOP_AT_DESTINATION

-> MISSION_COMPLETE

FAILSAFE states:

COMM_LOSS

DESTINATION_TIMEOUT

ESTOP

DRIVE_FAULT

---

# MAVLink and ArduPilot behavior required

## UAV control rules

Use proper ArduPilot mode changes and arming flow.

Use guided/takeoff/landing APIs rather than RC override as the primary strategy.

Use Precision Landing via **MAVLink `LANDING_TARGET`** for the moving UGV landing phase.

Send `LANDING_TARGET` at high rate during terminal guidance.

Prefer filling `x`, `y`, `z` body-frame fields with `position_valid=1` when full pose is known.

## Important ArduPilot parameters to support in config

These should be present in config and documented in setup notes:

`PLND_ENABLED = 1`

`PLND_TYPE = 1` for MAVLink `LANDING_TARGET`

`PLND_ORIENT`

`PLND_YAW_ALIGN`

`PLND_CAM_POS_X`

`PLND_CAM_POS_Y`

`PLND_CAM_POS_Z`

`PLND_LAND_OFS_X`

`PLND_LAND_OFS_Y`

`PLND_XY_DIST_MAX`

`PLND_STRICT`

`PLND_ALT_MIN`

`PLND_ALT_MAX`

`PLND_OPTIONS` with moving-target support enabled

`FLOW_TYPE`

`FLOW_POS_X`, `FLOW_POS_Y`, `FLOW_POS_Z`

`RNGFND1_TYPE`

`RNGFND1_MIN`

`RNGFND1_MAX`

`RNGFND1_ORIENT`

`LAND_SPD_MS`

## Optical flow and CAN assumptions

For a DroneCAN optical flow unit, support config patterns such as:

`FLOW_TYPE = 6` if using DroneCAN flow

`CAN_P1_DRIVER = 1`

`CAN_D1_PROTOCOL = 1`

## LidarLite v3 caution

Treat LidarLite v3 integration as bench-test critical. The code and docs must note:

PWM is preferred over I2C if available due to documented I2C issues.

The rangefinder must be validated on the bench and in logs before flight use.

---

# Vision implementation requirements

## Destination marker detection

Implement a module that:

Grabs frames from the ZED X.

Detects the destination ArUco marker.

Estimates its pose relative to the camera using calibrated intrinsics.

Converts the observation into a destination payload suitable for the UGV.

Logs confidence, marker ID, timestamp, and relative pose.

## Moving UGV landing marker detection

Implement a separate tracking module that:

Detects the landing marker on the UGV deck.

Estimates relative pose at real-time rate.

Filters jitter using a configurable filter.

Rejects outliers and stale detections.

Converts target pose into `LANDING_TARGET` messages.

Continues feeding ArduPilot until touchdown or target loss.

## Required helper modules

camera calibration loader

image-to-body-frame transform

marker-size-aware pose estimator

temporal filter (EMA or Kalman)

confidence gate

target-loss detector

---

# Radio / direct communication requirements

Build on the uploaded `v2v_bridge.py`, `esp_uav.cpp`, and `esp_ugv.cpp`, but clean them up.

## Keep the same wire protocol concept

Start byte: `0xAA`

Frame fields: `type`, `len`, `payload`, xor checksum

Packet families:

- telemetry

- commands

- debug/status text

## Add formal message schemas

Define explicit payloads for:

#### UAV -> UGV

heartbeat

destination found

destination pose

mission phase

estop

touchdown predicted

touchdown confirmed

#### UGV -> UAV

heartbeat

motion state

speed

yaw / heading if available

destination ack

deck ready

estop

## Reliability requirements

sequence numbers

duplicate rejection

heartbeat timeout

reconnection handling

telemetry timestamping

mission event logging

---

# Safety and failsafe requirements

Implement all of these explicitly.

## UAV failsafes

MAVLink link loss

radio link loss to UGV

camera stream failure

landing marker lost during descent

rangefinder unhealthy or stale

optical flow unhealthy

EKF unhealthy

low battery hook

emergency land command

## UGV failsafes

loss of destination message

loss of UAV heartbeat

estop from UAV or operator

obstacle avoidance timeout

drive controller fault

## Behavior rules

If the landing target is lost above the configured threshold, hold or retry based on mission config.

If the target is lost below the configured threshold, allow controlled vertical landing only if explicitly configured.

If UGV heartbeat is lost during final approach, abort landing and climb/hold if safe.

If any critical health monitor fails, transition to a safe state and log it.

---

# Exact coding tasks for the AI

The AI should write all of the following.

## 1. Config system

Strongly typed config loader.

YAML-based configs for airframe, camera, markers, mission, radio, and UGV assumptions.

Validation for missing keys and invalid values.

## 2. MAVLink wrapper

Robust Python MAVLink client.

Connection retry logic.

Heartbeat wait.

Mode change helper.

Arm/disarm helper.

Takeoff helper.

Land helper.

`LANDING_TARGET` sender.

Health subscribers for EKF, distance sensor, optical flow, battery, and mode.

## 3. ArUco vision stack

OpenCV-based detector.

Camera calibration loading.

Pose estimation using marker size.

Two logical trackers:

- destination marker tracker

- landing marker tracker

Jitter filtering and outlier rejection.

## 4. Precision landing module

Converts filtered target pose into MAVLink `LANDING_TARGET`.

Supports angle-only mode and full-position mode.

Supports moving-platform landing.

Runs in a timed loop with configurable rate.

## 5. UAV mission manager

Full UAV state machine.

Timeouts for each phase.

Structured logs.

Event callbacks.

Mission result summary.

## 6. UGV mission manager

Receives destination packet.

Sends ACK.

Starts motion.

Exposes drive commands.

Handles obstacle avoidance hooks.

Sends deck-ready and motion telemetry.

## 7. ESP32 firmware cleanup

Rewrite both ESP32 programs into clean production code.

Preserve protocol compatibility where practical.

Add heartbeat frames and status LEDs.

Add compile-time config for peer MAC addresses.

Add serial diagnostics.

## 8. Testing tools

Mock video replay for ArUco testing.

Radio protocol test script.

Bench `LANDING_TARGET` publisher for SITL.

Log replay utility.

Unit tests for protocol and state machines.

## 9. Documentation

setup guide

wiring guide

parameter checklist

field test checklist

requirements matrix

test matrix mapped back to competition requirements

---

# Recommended message definitions

Use dataclasses or pydantic models in Python and packed structs on ESP32.

## Example: destination message

class DestinationFound:

seq: int

timestamp_ms: int

marker_id: int

x_m: float

y_m: float

z_m: float

yaw_rad: float

confidence: float

## Example: UGV motion state

class UgvTelemetry:

seq: int

timestamp_ms: int

speed_mps: float

yaw_rad: float

deck_ready: bool

estop: bool

mission_phase: int

## Example: landing target observation

class LandingObservation:

timestamp_ms: int

marker_id: int

x_body_m: float

y_body_m: float

z_body_m: float

angle_x_rad: float

angle_y_rad: float

confidence: float

valid: bool

---

# Implementation guidance for existing uploaded files

## `mission1.py`

Current status:

useful for showing rough mission ordering

not acceptable as final flight-control logic

uses RC throttle override and placeholder camera input

should be replaced by state-machine-driven ArduPilot control

## `ground_station.py`

Current status:

useful as a rough UGV movement and MAVLink control prototype

obstacle avoidance is placeholder only

should be refactored into modules with explicit state handling

## `v2v_bridge.py`

Current status:

useful protocol base

should be cleaned up, typed, and expanded with formal message schemas

## `esp_uav.cpp` and `esp_ugv.cpp`

Current status:

useful starting bridge firmware

should be cleaned up and documented

preserve the framing logic unless there is a strong reason to change it

---

# Minimum bench-test sequence

The generated project must include scripts and docs for this exact validation order.

## Bench phase 1: radio only

verify UAV↔UGV heartbeat

verify command/telemetry round trip

verify sequence numbers and timeout handling

## Bench phase 2: vision only

verify destination marker detection distance

verify landing marker detection distance

verify off-axis and lighting performance

verify pose estimator scale correctness

## Bench phase 3: autopilot only

verify flow health

verify rangefinder health

verify stable non-GPS hover near ground

verify mode changes and arming logic

## Bench phase 4: SITL / dry run

publish synthetic `LANDING_TARGET`

verify precision landing state transitions

verify moving-target handling logic

## Flight phase 1

stationary landing marker on the ground

## Flight phase 2

stationary UGV landing deck

## Flight phase 3

slow-moving UGV landing

## Flight phase 4

full challenge flow

---

# Wiring / integration notes to document in code comments and setup docs

## Cube Orange+

ArduCopter installed

Holybro H-Flow connected via CAN/DroneCAN

LidarLite v3 connected as downward rangefinder

Jetson Nano connected via telemetry UART/USB for MAVLink companion link

## Jetson Nano

ZED X downward camera

Python runtime for mission manager and vision stack

serial link to ESP32-S bridge

MAVLink link to Cube

## ESP32-S

one unit on UAV side

one unit on UGV side

ESP-NOW peer MAC addresses configurable in source or build config

---

# Coding quality requirements

Python 3.10+

type hints everywhere

no giant monolithic scripts

logging module, not print-only debugging

all constants moved into config

no hardcoded magic numbers except protocol constants

exception-safe cleanup

every long-running loop has timeout and health checks

every state transition logged with reason

---

# Deliverable quality bar

The AI must generate code that is:

modular

typed

documented

bench-testable

not dependent on GPS for the UAV landing phase

consistent with the uploaded competition rules

consistent with ArduPilot precision landing architecture

Do not generate toy pseudocode. Generate real files with runnable scaffolding and clear TODOs where hardware-specific values are needed.

---

# Final instruction to the coding AI

Use the uploaded files as references, but rewrite the system into a clean ArduPilot-first architecture. Preserve the useful radio bridge concept. Replace the RC-override UAV control pattern with proper ArduPilot companion-computer control and `LANDING_TARGET`-based precision landing. Build the project so it can be tested in stages and adjusted quickly on competition day.